# Fleet Message Bandwidth

Measured message sizes, rates and link budgets for Ekko, Crusader, Graey and the OCS

Team Inspiration · Maritime RobotX Challenge 2026 · Singapore

This document exists to size the links. Every byte figure in it was measured by encoding the actual message with realistic values, not estimated — protobuf for the RoboCommand link, JSON for the vehicle links, with the framing each one really carries.

# The answer, up front

| Link | Sustained | Peak | Verdict |
| --- | --- | --- | --- |
| OCS → RoboCommand | 7.4 kbit/s | 35.2 kbit/s | Trivial |
| Crusader ⇄ OCS (WiFi) | 5.9 kbit/s | 34.4 kbit/s | Trivial |
| Ekko ⇄ OCS (WiFi) | 5.9 kbit/s | 34.4 kbit/s | Trivial |
| Graey ⇄ Crusader (acoustic) | 0.46 kbit/s available | 1.2 kbit/s needed | DOES NOT FIT |

Three of the four links are not a constraint at any message rate the competition asks for — the whole fleet at peak is about 2% of a single MJPEG camera stream. The fourth is short by a factor of three, and no encoding choice closes that gap. Section 6 has the arithmetic.

# Contents

# 1. What is being counted

Each message crosses at most two links, and it is a different size on each because the two links encode differently.

|  | OCS ↔ RoboCommand | Vehicle ↔ OCS |
| --- | --- | --- |
| Encoding | Protocol buffers (binary) | JSON (text) |
| Chosen by | RoboNation — we cannot change it | Us |
| Transport | MQTT over TCP | TCP |
| Framing per message | 39 B (MQTT header + topic) | 4 B (length prefix) |
| Plus | 40 B TCP/IP | 40 B TCP/IP |

JSON runs roughly four times larger than the same message in protobuf. That is the deliberate cost of letting the vehicles carry no generated code, and on a WiFi link it buys a great deal of simplicity for bytes that are free. It is not free on the acoustic link, which is why nothing there uses JSON.

# 2. Message sizes, measured

Payload only — add the framing from section 1. Sizes assume realistic content: ten buoys in a Safe Passage Report (the handbook specifies ten), eight pipeline segments, a five-point geofence, three vehicles in the declaration.

| Message | To RC (bytes) | To OCS (bytes) | Rate |
| --- | --- | --- | --- |
| Heartbeat — Crusader (USV) | 76 | 327 | 2 per second, continuous |
| Heartbeat — Graey (UUV) | 76 | 319 | 2 per second, continuous |
| Heartbeat — Ekko (UAV) | 78 | 366 | 2 per second, continuous |
| Course | 119 | 304 | Once, held by the broker |
| Run Declaration | 144 | 448 | Once per run |
| Run Start | 22 | 105 | Once per run |
| Safe Passage Report (10 buoys) | 306 | 1144 | Resent whenever detection changes |
| Pipeline Survey Report (8 segments) | 55 | 394 | Once, on surfacing |
| Resource Delivery Request | 31 | 204 | On reading the light code |
| Docking Report | 27 | 105 | On docking |
| Firefighting Report | 27 | 113 | On extinguishing |
| Assistance Request | 40 | 167 | As incidents occur |
| Keep-Out Zone | 45 | 176 | As hazards appear |
| All Clear | 20 | 100 | When a zone is lifted |
| Moving Object Alert | 52 | 227 | As incidents occur |
| Readiness Confirm | 27 | 120 | Answering a readiness report |
| Incident Acknowledgement | 28 | 116 | Immediately on any Task 4 command |
| Readiness Report | 28 | 113 | On reaching station |

One message dominates everything else. The Safe Passage Report is four times the size of a heartbeat and is resent every time the buoy picture changes — continuously, at the Disruptive tier, where the buoy colours change in real time. It is the only message worth optimising.

## How the big messages scale

Two messages carry repeated elements, and they scale very differently:

| Element | Cost each | Why |
| --- | --- | --- |
| One more buoy | 24 bytes | Carries a position — two 8-byte coordinates plus framing. |
| One more pipeline segment | 1 byte | Carries only intact-or-damaged. Segment count is effectively free. |
| One more geofence corner | 20 bytes | A position, as above. |

So a twenty-buoy field would cost about 546 bytes, and a forty-segment pipeline still costs under 90. If a size budget is ever tight, buoy count is the number that moves it.

# 3. The OCS → RoboCommand link

Heartbeats are the only continuous traffic: two per second per vehicle, for the whole run. Everything else is occasional.

RoboNation caps the team at five messages per second per vehicle. Two are spent on heartbeats, leaving three for task reports — so the worst case is a vehicle sending three Safe Passage Reports a second alongside its heartbeats.

| Case | Bytes/s | kbit/s | Per |
| --- | --- | --- | --- |
| Heartbeats only, one vehicle | 310 | 2.5 | vehicle |
| Heartbeats only, three vehicles | 930 | 7.4 | fleet |
| At the 5 msg/s cap, one vehicle | 1,465 | 11.7 | vehicle |
| At the cap, three vehicles — size for this | 4,395 | 35.2 | fleet |

Size this link for 35 kbit/s. That is the ceiling the protocol itself permits — there is no message pattern that exceeds it without breaking the rate cap.

# 4. The vehicle ⇄ OCS links (WiFi)

Same messages, JSON instead of protobuf, so roughly four times the bytes. Still nothing a WiFi link notices.

| Case | Bytes/s | kbit/s | Per |
| --- | --- | --- | --- |
| Heartbeats only, one vehicle | 742 | 5.9 | vehicle |
| Heartbeats only, three vehicles | 2,226 | 17.8 | fleet |
| Two heartbeats + three reports, one vehicle | 4,306 | 34.4 | vehicle |
| Same, three vehicles — size for this | 12,918 | 103.3 | fleet |

Size these for about 35 kbit/s per vehicle, 105 kbit/s for the fleet. For comparison, one MJPEG camera at 640×480 and 15 frames per second is roughly 4,800 kbit/s — so all three vehicles at peak telemetry come to about one fiftieth of a single video stream. If the team WiFi is carrying video at all, telemetry is a rounding error against it.

# 5. Where the traffic actually goes

Both directions, so the link budget is complete:

| Direction | What travels | Share of the bytes |
| --- | --- | --- |
| Vehicle → OCS | Heartbeats and task reports | Essentially all of it |
| OCS → RoboCommand | The same, re-encoded and attributed | Essentially all of it |
| RoboCommand → OCS | Course once, run start once, Task 4 commands as incidents occur | Negligible — under 60 bytes each, event-driven |
| OCS → Vehicle | The same commands, passed on | Negligible |

The return direction never needs sizing. Everything inbound is event-driven and under 60 bytes, so even a busy Task 4 sequence adds well under 1 kbit/s.

# 6. The acoustic link — the one that does not fit

Graey has no path to the OCS except through Crusader. While it is submerged that path is an acoustic modem, and this is the only link in the fleet where bandwidth decides what is possible.

## What the modem gives

| Specification | Value |
| --- | --- |
| Raw data rate | 640 bit/s |
| Maximum throughput | 463 bit/s |
| Largest single message | 64 bytes |
| Airtime for a message of n bytes | 0.105 + (n + 16) × 0.0125 seconds |

Airtime is what matters, not the headline rate — the fixed overhead per message is large, so message count costs more than message size:

| Payload (B) | Airtime (s) | Max rate (Hz) | Note |
| --- | --- | --- | --- |
| 64 | 1.105 | 0.90 | The largest single message the modem accepts |
| 32 | 0.705 | 1.42 |  |
| 24 | 0.605 | 1.65 |  |
| 16 | 0.505 | 1.98 | Just short of 2 Hz |
| 15 | 0.492 | 2.03 | The largest payload that can reach 2 Hz at all |
| 8 | 0.405 | 2.47 |  |

## Against the requirement

The handbook requires a heartbeat at 2 Hz per active vehicle. A full UUV heartbeat is 76 bytes — already over the modem's 64-byte limit, so it must be split into two messages:

| Step | Value |
| --- | --- |
| First message (64 B) | 1.105 s |
| Second message (12 B) | 0.455 s |
| Total per heartbeat | 1.560 s |
| Achievable rate | 0.64 Hz — against 2 Hz required |

The gap is a factor of three, and it cannot be closed by encoding. Even a purpose-built binary heartbeat stripped to 15 bytes — position, depth, heading, state and nothing else — reaches 2.03 Hz only by occupying the channel completely, leaving no capacity for acknowledgements, for ranging, or for anything travelling the other way. The modem is half-duplex, so at full duty Graey could never receive.

This is a property of underwater acoustics, not of the hardware chosen. Across the entire published RoboSub 2026 field — 57 teams — the best demonstrated through-water rate is 12 bit/s; the commercial unit Graey carries is roughly forty times better than that and still three times short.

## What RoboNation expects

Their own schema anticipates the constraint. The pipeline survey report is defined as arriving only once the vehicle comes up:

Sent once, after the UUV completes both the survey and (Advanced tier) the repair — it can't transmit while submerged, so this typically isn't sent until it surfaces.  — RoboNation, report schema

The practical planning assumption: while Graey is down, expect position updates every few seconds rather than twice a second, and expect findings to arrive in a batch when it surfaces rather than as they are discovered. On the surface, WiFi or the tether is available and the constraint disappears entirely.

# 7. Two rules that affect what gets sent

## The relay does not change attribution or counting

Crusader carries Graey's reports, but they remain Graey's: attributed to Graey, and counted on Graey's own sequence counter rather than Crusader's. RoboNation states this explicitly, and makes no requirement about how a report reaches the OCS:

...scoped per vehicle (each vehicle's reports keep their own counter; not shared across the team, regardless of how the OCS relays them).  — RoboNation, report schema

## “Unknown” is not “stand down”

The task field in the heartbeat is how RoboCommand decides a task attempt has begun and ended. Moving to “none” means standing down; moving to “unknown” does not — that value also appears transiently during a mode change or when a field simply fails to be set. A vehicle that leaves it unset is sending a value the specification says must never be sent.

Sizes measured by encoding each message from robonation/robocommand, RobotX_2026/proto, revision f6457fa (6 August 2026). Acoustic figures from the Succorfish Delphis v3 / Nanomodem v3 datasheet and serial command manual.